CHAPTER  4

GENERAL  RULES

4.1.	WORLD CLASS RECORDS

4.1.1.	In the table "World Class and sub-class Records in Classes C, H, M and N" (Annex 1) the "x" indicates which records exist in the classes and sub-classes recognised.

4.1.2.	Only aeroplanes in Class H, Class M, Class N, and motorgliders may be used in record attempts in Class C. Records that have been homologated in Classes H, M, N, and R cannot be homologated in Class C.

4.1.3.	Motorgliders may be used for Class C records provided that the engine is kept running throughout the entire flight.

4.6.	DETERMINATION OF WEIGHT AND/OR WEIGHT CLASSIFICATION

4.6.1.	The greatest weight of the aircraft during the record attempt will determine its weight classification.

4.6.2.	If refuelling in flight occurs, the aircraft must be weighed prior to takeoff with all fuel tanks filled and sealed.  If refuelling in flight does not occur, the aircraft must be weighed prior to takeoff (in the same configuration that will be used during the record attempt), unless the record is being conducted under the provisions of 5.3.8., 5.3.9., or 5.4., 	 in which case the weight and balance records of the aircraft may be used in determining the weight.

4.8.	SHORT TAKE-OFF AND LANDING (STOL) AIRCRAFT RECORDS

4.8.1.	To qualify as a STOL aircraft, the take-off and landing of the record must be completed within a measured maximum distance of 500 metres. The effective headwind component at the time of take-off shall not be greater than 20 kilometres per hour.

4.10.	MEASURING EQUIPMENT

4.10.1.	Unless FAI General Aviation Commission has determined otherwise any measuring device previously used in any other FAI Air Sport or record may be used in support of record attempts under Class C, H, M, and N.

4.11.	MISCELLANEOUS

4.11.1.	No fuel, ballast or other disposable matter may be jettisoned after take-off and prior to the completion of the record attempt.

4.11.2.	No person may parachute from the aircraft during a record attempt.

CHAPTER 5

SPECIAL  RULES  FOR  WORLD  RECORDS

5.1.	DISTANCE RECORDS

5.1.1.	Distance without landing

5.1.1.1.	The course shall be approved or declared.

5.1.1.2.	The course shall not be a closed circuit.

5.1.1.3.	Turn points are allowed, providing that the average distance of each leg is a minimum of 5 000 kilometres.

5.1.1.4.	Each turn point must not exceed 90° in change of direction.

5.1.1.5.	Turn points must be overflown.

5.1.1.6.	Each leg will be measured.

5.1.1.7.	Refuelling in flight is not allowed.

5.1.2.	Distance over closed circuit without landing

5.1.2.1.	The course shall be an approved or declared course measured from the start point with a return to the start point.

5.1.2.2.	The take-off place and landing place shall be within a maximum of 10 kilometres of the start point and will be outside the perimeter of the closed circuit.

5.1.2.3.	Refuelling in flight is not allowed.

5.3.3.	Speed over a closed circuit, without landing. Course lengths of 100, 500, 1 000, 2 000, 5 000 and then by increases of 5 000 kilometres without payload

5.3.3.1.	The flight shall be made over an out-and-return course (with or without an intermediate turnpoint), which must be approved or declared. The distance may be achieved over more than one lap of the course.

5.3.3.2.	Before crossing the start line the aircraft shall fly level for the last 1 000 metres within a tolerance of 100 metres.

5.3.3.3.	The altitude of the aircraft at the finish line shall not be less than its altitude at the start line.

5.3.3.4.	Refuelling in flight may only be accomplished prior to the aircraft reaching the start line.

5.3.4.	Speed over a closed circuit, without landing. Course lengths of 100, 500, 1 000, 2 000, 5 000 and 10 000 kilometres with payload

5.3.4.1.	As in 5.3.3.1. - 5.3.3.4. inclusive.

5.3.4.2.	As in 5.2.4.2., 5.2.4.3. and 5.2.4.4. above.

5.3.5.	Speed around the world non-stop and non-refuelled

5.3.5.1.	The course, including the control points, shall be approved in advance by the NAC's concerned. It must start and finish at the same aerodrome, crossing all meridians. The length of the course shall not be less that 36 787.559 kilometres (equal in length to the Tropic of Cancer). The pilot may deviate as is necessary for safety or weather conditions provided appropriate control points can be established.

5.3.5.2.	All control points must be at a latitude of less than 66° 33’ (outside the North and South Frigid Zones).

5.3.5.3.	If due to unforeseen circumstances the final landing cannot be made at the aerodrome of departure, the aircraft may fly to an alternate aerodrome lying beyond the original one (at a greater distance from which the start was made).

5.3.5.4.	The start time shall be the time of take-off ; the finish time shall be the time of landing.

5.3.6.	Speed around the World (Eastbound and Westbound)

5.3.6.1.	The course, including the control points, shall be approved in advance by the NAC's concerned. It must start and finish at the same aerodrome, crossing all meridians. The length of the course shall not be less that 36 787.559 kilometres (equal in length to the Tropic of Cancer).

5.3.6.2.	All control points must be at a latitude of less than 66° 33’ (outside the North and South Frigid Zones).

5.3.6.3.	If due to unforeseen circumstances the final landing cannot be made at the aerodrome of departure, the aircraft may fly to an alternate aerodrome lying beyond the original one (at a greater distance from which the start was made).

5.3.6.4.	Intermediate landing places, turn or control points must be declared and agreed in advance with the NAC's concerned.

5.3.6.5.	Any time spent on the ground between start and finish shall be counted as flying time.

      - Refuelling on the ground or in flight is permitted.

5.3.6.7.	Repairs or replacements of aircraft components and engine(s) are permitted except that the wings and fuselage may not be changed.

5.3.6.8.	Crew members may not be changed during the attempt. However, in the case of an emergency a crew member, other than the pilot-in-command, may leave but must not be replaced. Passengers may be changed during the attempt.

5.3.6.9.	The start time shall be the time of take-off ; the finish time shall be the time of landing.

5.3.6.10.	Record attempts will be categorised as follows:

Category A - Without refuelling in flight.

Category B - With refuelling in flight.

5.3.6.11.	In the case of an aircraft having the capability of refuelling in flight and attempting a record under Category A, adequate evidence must be provided that refuelling in flight did not occur at any time during flight.

5.3.7.	Speed around the world over both the Earth's poles

5.3.7.1.	The course, including the control points, shall be agreed in advance with the NAC's concerned and must start and finish at the same aerodrome. The aircraft must overfly both geographic poles and the official controlling the attempt must be satisfied in advance as to how the necessary evidence is to be obtained.

5.3.7.2.	As in 5.3.6.3. - 5.3.6.11. inclusive.

5.3.8.	Speed over a Commercial Air Route

5.3.8.1.	The record attempt shall be flown on a normally scheduled and timetabled flight over a route regularly served by scheduled air services, recognised by IATA. Except on international flights, the route shall be at least 400 kilometres in length. Payload shall be carried during the attempt.

5.3.8.2.	The attempt may be made on the outward or return journey or can be for the double journey.

5.3.8.3.	In attempting the record for the double journey the time spent on the ground between outward and return journeys shall be counted as flying time. Conditions stated in 5.3.6.7. above shall apply.

5.3.8.4.	The flight shall be timed from take-off to landing and any time spent on the ground between the start and the finish shall be counted as flying time.

5.3.8.5.	The crew other than the pilot-in-command may be changed.

5.3.9.	Speed over a Commercial Air Route in excess of 6 500 kilometres

5.3.9.1.	The record attempt shall be flown on a normally scheduled and timetabled flight over a route regularly served by scheduled air services, recognised by IATA. The distance between start and finish points excluding intermediary points will not be less than 6 500 kms.

5.3.9.2.	The attempt will be made on the outward or return journey but will not count for the double journey.

5.3.9.3.	The flight will be timed from take-off to landing and any time spent on the ground between the start and the finish shall be counted as flying time.

5.3.9.4.	Conditions as in 5.3.6.7. above shall apply.

5.3.9.5.	The crew may be changed at any of the intermediate stops. Each pilot-in-command shall hold a valid FAI Sporting Licence and will be named on the record certificate and will be regarded as one of the holders.

5.4	SPEED OVER RECOGNISED COURSES

5.4.1.	Speed over recognised course records may be flown between any two cities and/or geographical features designated for that purpose by the NACs of the countries in which they are situated.

5.4.2.	The minimum distance between the start point and the finish point when these are situated in the same country shall be 400 kilometres. In countries where the NAC has not designated cities or geographical features - or points not designated which are to be used - the NAC organizing the attempt will be responsible for obtaining approval of the NAC(s) involved. FAI will be responsible for the approval of cities or geographic points not within Member countries.

5.4.3.	The start point and the finish point shall be situated within 60 kilometres of the city centre in such a way as the distance flown will not be less than the distance between the city centres, subject to a tolerance of 2% with a maximum of 60 kilometres. However when geographical features are used the start point and/or the finish point shall be situated at those geographical features.

5.4.4.	There shall only be one position recognised by FAI for each city or geographical point.

5.4.5.	Each course shall be the subject of a separate record for the journey out and the journey back, and for the journey out and back.

5.4.6.	Crews must not be changed.

5.4.7.	The speed recorded shall be the speed between the start and finish lines computed as the average speed along the great circle distance, any time spent on the ground being counted as flying time.

5.4.8.	For new records, the speed recorded must be equal to or greater than the minimum steady flight speed (stall speed with flaps up/Vs) of the aircraft.  If the minimum steady flight speed is not known, the speed recorded must be equal to or greater than 100 km/h.

5.4.9.	Repairs or replacement of aircraft components is permitted. Replacement of wing(s), fuselage and engine(s) is not permitted.